Modeling AND SiMULATION OF MagNETIC TRANSMISSION LINES

Muhammad Shamaas

ID # 2018-MS-EE-4

DEPARTMENT OF ELectrical Engineering

UNIVERSITY OF ENGINEERING AND TECHNOLOGY LAHORE

Contents

Abstract	2

Acknowledgements	3

List of Figures	4

List of Tables	4

No table of figures entries found.	4

Abstract	5

Introduction	6

1.1.	Nature of Magnetic Materials	6

1.2.	Magnetic Losses	10

1.3.	Summary	10

2.	Magnetic Circuit Modeling	11

2.1.	Reluctance Model	11

2.2.	Capacitance-Permeance Model	11

2.3.	Magnetic Transmission Line Model	13

2.4.	Conclusion	15

3.	Computational Electromagnetics	16

3.1.	Methods for solving Maxwell’s Equations	16

3.2.	MEEP	16

4. Simulations for Magnetic Transmission Lines	20

4.1. Wideband Transformer Design	20

4.2. Wideband Transformer Simulation in MEEP	20

4.	References	23

5.	Appendix	24

# Abstract

# Acknowledgements

# List of Figures

Figure 1: Magnetization vs. Applied Field	9

Figure 2: Hysteresis Loop	10

# List of Tables

# No table of figures entries found.

# Abstract

Magnetic Transmission Line is the dual counterpart of Electric Transmission Line. Its theory, encompasses a diverse range of applications including Transformers, Dynamic Machines, Microwave Generators, Tuners, Couplers, Isolators, Power Dividers etc. Intrinsically, Magnetic Transmission Line is made from a non-conducting magnetic material, with a high permeability. It transmits Magnetic Flux which acts as the Magnetic charge. Time varying magnetic flux results in a Magnetic Displacement Current inside the Transmission Line, which has the units of Volts. This produces a gradient Magnetic Field; with Fields Lines that spread radially outwards. The magnetic voltage due to this Magnetic Field is measured in Amperes. Although, the operation of a Magnetic Transmission Line does not involve electric charges, Magnetic Displacement Current produces an Electric Field with closed Field Lines encircling the Magnetic Transmission Line. Together, the Electric and Magnetic Fields transmit Energy along the direction of propagation. These relations will be modeled using Maxwell’s Equations and magnetic circuits to study the time and frequency domain behavior of Magnetic Transmission Lines. Furthermore, Finite Difference Time Domain Electromagnetic Field Simulations will be carried out in MEEP Simulator for anisotropic, inhomogeneous, non-linear Magnetic Transmission Lines.

# Introduction

Magnetic Transmission Lines are designed to transmit electromagnetic energy using strong magnetic fields. They are made of magnetic materials with very high magnetic permeability and a strong affinity for magnetic flux. When an external magnetic field is applied, atomic spins tend to align parallel to it. This large scale cooperation enhances the Magnetic Flux Density inside the magnetic material. When the applied field is varied, the changing Magnetic Flux Density transmits the magnetic information across the magnetic material. This phenomenon is called Magnetic Transmission.

It is important to note that charge carriers are not involved in magnetic communication. Isolated Magnetic charges do not exist and magnetic conduction current can never flow in a Magnetic Transmission Line. Magnetic Transmission is only possible through Magnetic Displacement current. This results from the alignment of magnetic dipoles in response to a stimulating Magnetomotive Force.

Moreover, Magnetic Transmission Lines do not involve the flow of Electric charges. Magnetic materials are very poor electric conductors; hence electric currents cannot transmit information across a Magnetic Transmission Line. Changing Magnetic Fields produce Electric Fields which are transmitted through electric displacement currents. This causes polarization of atoms in the dielectric magnetic medium which transmits Electric information across the Magnetic Material. Together, the Electric and Magnetic Fields transmit Electromagnetic Energy along the direction of propagation.

The following sections will elaborate on the subject of Magnetic Materials. A brief account on the losses in Magnetic Transmission Lines will be given as well.

## Nature of Magnetic Materials

The basic building blocks of magnetic materials are fictitious magnetic poles which can be considered as magnetic charge carriers. Poles can have either positive or negative charge. This is responsible for the magnetic field around the pole. Hence like poles repel each other and unlike poles attract each other. The force between poles is proportional to the strength of the poles (m) and inversely proportional to the square of distance (r) between them:

F=m1m24μ0r2                                                                         (1)

In nature, magnetic monopoles always exist in a pair called magnetic dipole. Magnetic dipole can result from the motion of an electron in an orbit around a nucleus; or its spin (either clockwise or anticlockwise) around an axis. This is similar to a current flowing in a loop. The identification of poles is dictated by the Flemming’s right hand rule. The magnetic moment (μ) of an orbiting/ spinning electron is proportional to the spectroscopic splitting factor (gs) and the associated quantum number (n). It is measured in units of Bohr Magneton (1μB=9.274×10-24JT-1).

μ=gsn                                                                          2

where g=1, n=1 for orbital moment (lowest state)

g=2, n=12                       for spin moment

The net magnetic moment of an atom or ion is the vector sum of the orbital and spin moments of all electrons in its outer shell.

Two dipoles attract each other if unlike poles are close to each other. On the other hand, two dipoles repel each other if like poles are closer. Hence, an external magnetic field can cause a mechanical torque and mechanical moment on a magnetic dipole. This moment tries to turn the dipole in the direction that decreases the overall energy of the system. In equilibrium state, the resultant moment due to both poles is zero.

In an unmagnetized material, the magnetic dipoles are randomly oriented hence their fields cancel out. When an external Magnetic Field H is applied, the magnetic dipoles rotate to decrease the energy of the system. In this way, the electrons try to occupy the energy levels with the lowest energy.

Atoms contain orbitals with a fixed amount of energy for accommodating electrons. An electron with clockwise spin can pair with an electron having anticlockwise spin. The clockwise spin can cancel the effect of anticlockwise spin hence no magnetic moment will result. Only unpaired spins contribute to the net moment. The resulting spin and orbital moments add up to produce a net Magnetization Vector Field M inside the magnetic material. This field is proportional to the magnetic susceptibility of the material (χm):

M=χmH                                                                        (3)

The Magnetic Field inside a magnetic material can be represented by a flow of M and H lines. The number of lines passing through a region of space is called Magnetic Flux (equivalent to magnetic charge). Magnetic Flux Density represents the number of flux lines per unit area:

B=μ0(H+M)                                                                  (4)

Iron, Nickel and Cobalt contain 4, 3 and 2 unpaired electrons per atom respectively. Hence, the effect of Magnetization/ Polarization is very strong in these special elements and their alloys. Large scale cooperation between magnetic dipoles causes an enhanced Magnetic moment. Due to the high magnetic susceptibility, they are used in the production of Ferromagnetic and Ferrimagnetic materials.

The parallel alignment of magnetic dipoles causes the creation of magnetic domains to reduce the magnetic potential energy stored in the Magnetic Flux Lines. The Magnetic energy consists of the following:

- Magnetostatic Energy: The energy needed to place the magnetic poles in a specific geometric configuration e.g. magnetized state. Magnetostatic Energy Density is proportional to the width of the magnetic strip (d) and the value of applied Magnetic Field Intensity (H). The expression is given in (5)

E∝dH2                                                                   (5)

- Magneto-crystalline Anisotropy Energy: For crystalline structures with repeating atomic units, the domain magnetization tends to align along one direction more easily than other directions. Magneto-crystalline Anisotropy Energy is greater in hard direction as compared to the easy direction. It depends on the anisotropy constants (Ki) and direction cosines (αi) which project magnetization on the different axes.

E=K1α12α22+α22α32+α32α12+K3α12α22α32…                      (6)

- Magnetostrictive Energy: Magnetization and Demagnetization can cause changes in the dimensions of the magnetic materials. These stresses are caused by shifting of atomic planes e.g. during alignment of domains. Magnetostrictive Energy represents the elastic potential energy stored in the constricted atomic configuration. It is proportional to the magnetostriction constant (λ) and applied stress (σ).

E∝32λσ                                                                  (7)

- Domain Wall Energy: A Domain wall is a region where the Magnetization in one domain gradually changes to the direction of a neighboring domain. Domain Wall Energy represents the energy in the transition region. It is related to Anisotropy Constant (Ka), Curie Point (Tc) and atomic spacing (a).

E∝KaTca                                                               (8)

Naturally, the size and direction of magnetic domains is chosen to minimize the overall magnetic energy of the system. If an unmagnetized material is placed in an external magnetic field, the domains may have to align in a hard direction for Magnetization of the material. Work will be done to align the domains in the special configuration so that the preferable domains grow in size while the unfavorable domains shrink. This will involve displacement of atomic planes and domain boundaries. Hence the overall stored magnetic energy of the system will increase during magnetization.

When a demagnetized material is placed in an increasing Magnetic Field, the domain walls will start reversible movements and rotations. The Magnetization will start to increase slowly as shown in the Figure below. This corresponds to the elastic phase with minimum magnetic susceptibility. Later on, the domain wall motions increase greatly. Large scale irreversible atomic plane displacements correspond to the partial magnetism phase in magnetization curve. During this phase, the material exhibits the highest magnetic susceptibility. Soon the majority of domains get aligned with the magnetic field. In the last phase, a large amount of energy is needed to rotate the remaining domain magnetization hence the material exhibits a saturating magnetic susceptibility. At high fields, the induction saturates at Bmax.

Figure 1: Magnetization vs. Applied Field

If the applied field of the saturated material is decreased, the magnetic domains start to reverse their direction. Initially, the material exhibits a small magnetic susceptibility. This resistance results because the majority of domains are aligned in the easy direction. The favorable domains had shrunk during the magnetization. Work must be done to expand the favorable domain walls in the reverse direction. As a result, demagnetization does not follow the curve of the original magnetization. When the applied field is decreased further, the magnetic susceptibility of the material starts to increase as more domains start to align in the reverse direction.

The induction lags the applied field hence some remnant induction remains when applied field is reduced to zero. In order to demagnetize the material, some extra amount must be applied. This amount is called the coercive force. As the field keeps decreasing, the domains start aligning in the hard direction. Once all the domains have aligned, the material saturates in the reverse direction.

If the material is now magnetized again, the response will contain all the phases of demagnetization. The induced field will start to increase slowly, followed by a phase of large magnetic susceptibility and end on saturation. Hence magnetization and demagnetization result in a hysteresis loop.

Figure 2: Hysteresis Loop

The slope of the B-H curve is called permeability. It is closely related to the magnetic susceptibility χm.

μ=BH=μ01+χm                                                               (9)

When the material is saturated, the magnetic susceptibility becomes zero. Hence the permeability reduces to μ0. Besides Magnetic Field Intensity, permeability is strongly dependent on chemical composition, crystal structure, stress, temperature and time after magnetization.

## Magnetic Losses

Grain and Grain Boundary

## Summary

# Magnetic Circuit Modeling

## Reluctance Model

- H. A. Rowland’s Law (1873) is the counterpart of G. Ohm’s Law (1827) for Magnetic circuits. Complex Reluctance Model defines Magnetic reluctance as the ratio of sinusoidal Magnetomotive Force and sinusoidal Magnetic Flux.

Rm=Fmϕm=H.dlB.dS=|Rm|ej∅

- Lossy Complex Magnetic Reluctance is non-linear and varies with the magnetic field. It resists both Magnetic flux and changes in Magnetic flux.

- In 1969, R. W. Buntenbach proved that the model is not power invariant. Reluctance Power Loss cannot be calculated using Joule Heating Law (1842) Analogy due to dimensional inconsistency:

[Pe]=Ie2Re=Ampere.Volt

but

[Pm]≠ϕm2|Rm|=Volt.Second.Ampere

- Hence this is not an accurate model for Power and Energy Flow.

## Capacitance-Permeance Model

- B. Tellegen’s Gyrator theory (1948) can describe power invariant transformation of magnetic and electric quantities. The dual effort and flow quantities are related by the gyration constant (N). R. W. Buntenbach proposed Power Invariant Permeance-Capacitance Model (1969) to replace Reluctance Model.

- Magnetic Displacement Current is the rate of change of Magnetic Flux which results from the polarization of Magnetic Dipoles. For a magnetic core, the magnetic current and Magnetomotive Force are given by:

Im,disp=dϕmdt=-1NVe [Volt]

and

Vm=Fm=NIe          [Ampere]

- Magnetic Permeance is defined as:

Pm=ϕmFm=B.dSH.dl=μAl [Henry]

This represents an equivalent magnetic capacitor which stores magnetic charge (magnetic flux).

- M. Faraday’s Law (1831): Electric Voltage is responsible for producing Magnetic Current (rate of change of magnetic flux).

Ve=-Ndϕmdt

E.dl=-ddtB.dS

- A. Ampere’s Law (1861): Magnetic Voltage is responsible for producing Electric Current (rate of change of electric flux).

Fm=Ndϕedt

H.dl=ddtD.dS

## Magnetic Transmission Line Model

- J. A. B. Faria and M.P. Pires presented Magnetic Transmission Line Model (2012) based on Electric Transmission Line Model in terms of per unit length transverse Impedance and per unit length Longitudinal Admittance.

- Per unit length Magnetic Conductance, Magnetic Inductance and Magnetic Capacitance are defined as:

Gm=FmIm=H.dlE.dl        [Ohm/m]

Lm=ϕmFm=B.dSH.dl      [Henry/m]

Cm=ϕeIm=D.dSE.dl       [Farad/m]

- Energy is dissipated in Magnetic Conductance due to skin effect.

Ploss=Im2Gm

- Electrical Energy is stored in Magnetic Capacitance; and Magnetic Energy is stored in Magnetic Inductance.

We=12CmVm2=12B.H dV

Wm=12LmIm2=12D.E dV

- The Magnetic Transmission Line Equations can be solved just like Electric Transmission Line Equations.

| Electric Transmission Line | Magnetic Transmission Line |
| --- | --- |
| dIedz=-CdVedt dVedz=-LdIedt | dImdz=-LmdVmdt dVmdz=-CmdImdt |
| d2Iedz2=γ2d2Iedt2 d2Vedz2=γ2d2Vedt2 | d2Imdz2=γ2d2Imdt2 d2Vmdz2=γ2d2Vmdt2 |
| Vez=Vei(0)e-γz+Ver(0)e+γz Iez=Iei0e-γz-Ier(0)e+γz | Vmz=Vmi(0)e-γz+Vmr(0)e+γz Imz=Imi0e-γz-Imr(0)e+γz |
| γ=jωlc=jωμε=jβ | γ=jωLmCm=jωμε=jβ |

| Electric Transmission Line | Magnetic Transmission Line |
| --- | --- |
| dIedz=-GVe-CdVedt dVedz=-RIe-LdIedt | dImdz=-LmdVmdt dVmdz=-GmIm-CmdImdt |
| d2Iedz2=γ2d2Iedt2 d2Vedz2=γ2d2Vedt2 | d2Imdz2=γ2d2Imdt2 d2Vmdz2=γ2d2Vmdt2 |
| Vez=Vei(0)e-γz+Ver(0)e+γz Iez=Iei0e-γz-Ier(0)e+γz | Vmz=Vmi(0)e-γz+Vmr(0)e+γz Imz=Imi0e-γz-Imr(0)e+γz |
| γ=(r+jωl)(g+jωc)=(ρ+jωμ)(σ+jωε)=α+jβ | γ=(jωLm)(Gm+jωCm)=(jωμ)(σ+jωε)=α+jβ |

The generator-receptor Non-Linear Magnetic circuit model is well suited for studying Electromagnetic Interference and Electromagnetic Compatibility of Magnetic Transmission Lines.

A

## Conclusion

|  | Reluctance Model | Permeance-Capacitance Model | Transmission Line Model |
| --- | --- | --- | --- |
| Conserved Quantity | ? | Magnetic Flux [Volt-Second] | Magnetic Flux [Volt-Second] |
| Flow Variable | Magnetic Flux [Volt-Second] | Rate of change of Magnetic Flux [Volt] | Rate of change of Magnetic Flux [Volt] |
| Effort Variable | Magnetomotive Force [Ampere] | Magnetomotive Force [Ampere] | Magnetomotive Force [Ampere] |
| Energy Dissipation Element | Magnetic Reluctance [Henry-1] | ? | Magnetic Conductance [Ohm] |
| Electrical Energy Storage Element | ? | ? | Magnetic Capacitance [Farad] |
| Magnetic Energy Storage Element | ? | Magnetic Permeance [Henry] | Magnetic Inductance [Henry] |

# Computational Electromagnetics

## Methods for solving Maxwell’s Equations

## MEEP

MEEP (2006) is a script based Simulator for modeling the time domain and frequency domain behavior of a variety of arbitrary materials including anisotropic, dispersive, non-linear dielectrics, electric/ magnetic conductors, media with saturable gain / absorption, and gyrotropic media.

- C++ interface: Features variable resolution and normalized units.

- Material Library: Sample data for several materials is provided in libraries for building accurate test structures.

- Current Sources: A wide variety of electric or magnetic current sources can be simulated.

- Derived components: Electric/ Magnetic/ Thermal Energy Density, Poynting Flux etc. can be evaluated.

- Mathematical operations: Averaging, symmetry and integration are allowed in cylindrical and rectangular coordinates.

- Data Visualization: The fields can be printed as image or video files.

- A. Ampere’s Law (1861)

∇×H-JD-σDD=dDdt

- M. Faraday’s Law (1831)

-∇×E-JB-σBB=dBdt

- J. C. F. Gauss’s Law for Electricity (1813)

∇.D=ρe

- J. C. F. Gauss’s Law for Magnetism (1813)

∇.B=ρm

- K. S. Yee’s Method (1966) or Finite Difference Time Domain Method is a differential numerical modeling technique for computational electrodynamics.

- J. C. Maxwell’s Equations (1861) are discretized using central difference approximations to the space and time partial derivatives. For example,

∇×H=∆Hz∆y-∆Hy∆zax+∆Hx∆z-∆Hz∆xay+∆Hy∆x-∆Hx∆yaz

∆Hz∆y=Hznx,y+∆y2,z-Hzn(x,y-∆y2,z)∆y

where n represents the discrete time step.

- Finite Difference Time Domain Method discretizes space into a grid of small elements called Yee Lattice (1966). The different field components at a grid location are stored in the edges and faces of a cubic element. They are evolved in discrete time steps.

The finite region of space must always be terminated with some boundary conditions. Three types of terminations are supported:

- Bloch-periodic Boundaries: These are used for simulation of periodic structures fx=f(x+L)e-jkL. Periodic Bloch Boundaries copy the field component at one cell’s edge and reinject them at a neighboring cell’s edge.

- Metallic Walls: All fields are forced to be zero at the boundaries (perfect reflector has zero absorption and zero skin depth).

- Perfectly Matched Layers: All the fields pass through the open boundary with no reflection. These absorbing boundary layers (ABC) absorb all incident fields.

- ε(x) and μ(x) can vary with position inside a material. They can be declared at each individual point x in space using a function.

- 1-D, 2-D and 3-D simulation is possible. Hence every space vector can have up to three spatial coordinates.

- The simulation can be carried out in rectangular or cylindrical coordinates. Hence different homogeneous/inhomogeneous structures can be built inside the space.

- Symmetry can be used to create complex geometries as well.

- Drude-Lorentzian Model (1900) models frequency dependent permittivity and permeability. Flux Densities contain terms for infinite frequency response and frequency dependent Polarization vector.

D=ε∞E+P

B=μ∞H+M

- ε and μ are represented as a sum of harmonic resonances and a term for frequency independent electric conductivity.

εω,x=(1+jσDω)(ε∞x+Nσnxωn2ωn2-ω2+jωγn)

μω,x=(1+jσBω)(μ∞x+Nσnxωn2ωn2-ω2+jωγn)

σD/σBis the electrical/magnetic conductivity. σN couples the polarization to the driving field, ωN is the angular resonance frequency, γN is a damping factor.

- Dispersion Drude-Lorentzian Model (1900) explains the electrodynamic properties of metals by regarding conduction band electrons as non-interacting electron gas.

- When the material is excited by an external source of resonant frequency, the material absorption loss increases greatly. Electromagnetic Energy is converted into other forms of energy.

- In Pockels and Kerr Non-linearity model (1875), ε and μ can be changed by the field intensity.

D=(ε∞x+χ2x.diagE+χ3x.|E|2)E+P

B=(μ∞x+χ2x.diagH+χ3x.|H|2)H+M

χ2 sum is the Pockels effect; whereas χ3 sum is the Kerr effect.

- Ferromagnetic materials are non-linear as their permeability varies with the strength of applied field intensity.

- At high magnetic field intensity, the material saturates, limiting further increase of Magnetic Flux. Hence, the susceptibility decreases rapidly.

- Landau-Lifshitz-Gilbert model (1955) describes the precessional motion of saturated magnetic dipoles in a magnetic field.

dMndt=bn×-σnH+ωnMn+αndMndt-γnMn

Mndescribes the linear deviation of magnetization from its static equilibrium value. Precession occurs around this unit bias vector bn. σn couples the polarization to the driving field, ωn is the angular resonance frequency, γn is a damping factor.

- For such anisotropic media, non-diagonal susceptibility tensor is used to relate Magnetization and Field intensity.

Mn=χ⊥-jη0jηχ⊥000χ||H

- G. Green’s Functions (1835) give the Field Patterns from a localized point source at a particular frequency ω.

Gij(ω;x,x')

The point current source j is placed at x'. The ith field component is observed.

Jx=ej.e-jωt.δ(x-x')

- A frequency domain solver is also provided for multidimensional Fourier transformation (1822) and the decomposition of fields into travelling modes.

- Broadband response: The 3 Dimensional Discrete Fourier transform (1822) of the response to a short impulse can give useful information about the transmitted power and losses.

- The Transmitted Power can be computed using the integral of Poynting Vector (1884); over a surface on the far end of the transmission line.

Pω=Re {n.Eωx*×Hωxd2x}

- Transmitted power and incident power can be used to find power losses in transmission line.

# Magnetic Transmission Line Simulation

## 4.1. Wideband Transformer Design

## 4.2. Wideband Transformer Simulation in MEEP

- The Magnetic Transmission Lines will be constructed for inhomogeneous, dispersive, non-linear ferromagnetic conductors like Ferromagnetic, Permalloy and Cobalt alloys.

- The Transmission Lines will be excited using continuous current sources.

- The terminations can be modeled by Perfectly matched layers for complete absorption; or as perfect reflectors for no load.

- Different Transmission Line structures can be simulated like shielded transmission line and multi-wire transmission lines.

- The multi-dimensional discrete Fourier transform (1822) and mode decomposition will be used to determine the Absorbance, Transmittance and Broadband Response.

- A wideband transformer passes a frequency band of several decades and are usually designed to handle complex waveforms like rectangular pulses. They are used for impedance matching, voltage/ current transformation, DC isolation, mixing, power splitting, coupling and signal inversion.

- A wideband transformer will be simulated. It will be excited by a small pulse to examine the Frequency Response. The 3 dimensional discrete Fourier Transform will be used to determine Absorbance, Transmittance and Broadband Response. The results can be compared with published datasheet.

The Loss tangent tanδ has the following components:

- DC Resistance Loss Tangent tanδdc

- Skin Effect Loss Tangent tanδse

- Proximity Effect Loss Tangent tanδpe

- Self Capacitance Dielectric Loss Tangent tanδcp

- Self Capacitance Circulating Currents Loss Tangent tanδcs

- Core Residual Loss Tangent tanδr

- Core Eddy Current Loss Tangent tanδf, Pe=(πfτBmax)2V6ρ

- Core Hysteresis Loss Tangent tanδh, Ph=ηVfBmaxn

Non-linear components must be used for these complex effects. Network Equivalent Magnetic circuits and coupled equations will be used to simplify analysis of the transient and steady state behavior.

- Magnetic coupling between magnetic transmission lines results in sharing of electromagnetic energy. This division of power is very useful in design of Radio frequency devices like sensors, antennas and communication systems.

- Magnetic Coupling is also very important in the working of DC and AC machines like induction motor, hysteresis motor and Reluctance motor.

- The study of capacitive/ inductive coupling in Multi-Conductor Transmission Lines will provide useful knowledge about the Radiated/ Conducted Emissions and Radiated/ Conducted Susceptibility.

- The results can be compared with MATLAB linear circuit models for cross talk between Magnetic Transmission Lines.

- The aim will be to minimize Electromagnetic Radiation; that can be picked up by unintentional receivers like digital Computers.

The simulators can not be used to model the following magnetic effects:

- Magnetostriction

- Accoustic effects

- Relativistic Effects

- Magnetohydrodynamics

- Gravitomagnetism

- Optical Effects

# Conclusion

The conventional reluctance model is not accurate for the modeling of magnetic circuits. It must be replaced by Magnetic Transmission Line Model for accurate modeling of such inhomogeneous, dispersive, non-linear structures.

The power invariant Magnetic Transmission Line model can also be used for accurate modeling of

- AC and DC Machines

- Micro-strip Antennas

- Waveguides

- Gyromagnetic NLTLs

# References

# Appendix